ERROR VMX DOESN'T SEE THE FPC:

---Unresponsive---

TSHOOT

JTAC REQUIRES:Red Hat Enterprise Linux 7.2 ; Linux 3.10.0-327.4.5 or later ; QEMU-KVM 1.5.3 or later

/usr/libexec/qemu-kvm --version QEMU emulator version 1.5.3 (qemu-kvm-1.5.3-167.el7_7.1)

https://forums.juniper.net/t5/vMX/vMX-can-t-connect-fpc-slot-0-Absent/td-p/298032

"set chassis fpc 0 lite-mode"

VPC/VFP requirements

ping 128.0.0.16 routing-instance __juniper_private1__   # from the VFC

VFP: This should be present in /var/log/messages

RPIO: Accepted connection from 128.0.0.1:50896 <-> vPFE:3000

RPIO: Accepted connection from 128.0.0.1:56098 <-> vPFE:3000

vagrant@vmx> show chassis fpc

Temp  CPU Utilization (%)   CPU Utilization (%)  Memory    Utilization (%)

Slot State            (C)  Total  Interrupt      1min   5min   15min  DRAM (MB) Heap     Buffer

0  Offline         ---Unresponsive---

vagrant@vmx> request chassis fpc slot 0 restart

restart chassis-control

Restart initiated, use "show chassis fpc" to verify

vagrant@vmx> show chassis fpc

Temp  CPU Utilization (%)   CPU Utilization (%)  Memory    Utilization (%)

Slot State            (C)  Total  Interrupt      1min   5min   15min  DRAM (MB) Heap     Buffer

0  Present          Absent

>>> APIC disabled. if still fails, optimize huge pages as per: https://www.juniper.net/documentation/en_US/vmx/topics/topic-map/vmx-installing-on-kvm.html#id-preparing-the-ubuntu-host-to-install-vmx

grubby --update-kernel=ALL --args="default_hugepagesz=1G hugepagesz=1G hugepages=32"

yum install grub2-efi-modules

[root@oracletest.cambridge.grapeshot.co.uk vmx-test]# grub2-install /dev/sda1

reboot

grep Huge /proc/meminfo << to see the hugepages sizes and number in RHEL

=============

------

SYSTEM CONFIGURATION:

Host operating system

Red Hat Enterprise Linux Server release 7.7 (Maipo)

Linux kernel:

Linux oracletest.cambridge.grapeshot.co.uk 4.14.35-1902.5.2.2.el7uek.x86_64

QEMU-KVM version

]# /usr/libexec/qemu-kvm --version

QEMU emulator version 1.5.3 (qemu-kvm-1.5.3-167.el7_7.1), Copyright (c) 2003-2008 Fabrice Bellard

Vagrant version

# vagrant version && vagrant plugin list

Installed Version: 2.2.5

vagrant-libvirt (0.0.45, global)

=====

LICENSE:

E435890758 aeaqic aiagij cpabsc idycyi giydco bqgiyd

sdapoz gvqlkk ovxgs4 dfojcx mylma4 uhfs7p

etdlfc ex2i5k 4mo4fa ew2xbz ujzbol 4fl44z

ya3md2 drdgjf preowc 5ubrju kyq